Signals of RAILSIMâ - Version USA 1.21 ã1998 by Jens Schubert:

1. „Clear“: Vmax allowed (track speed, train speed)

1*)2*)3*)	1*)2*)	1*)2*)	1*)	1*)2*)3*)

...at next signal:	clear		limited		medium		slow		stop

2. „Limited“ : Vmax. 45 mph

...at next signal: 	clear		limited		medium		slow		stop

3. „Medium“ : Vmax. 30 mph

...at next signal:	clear		limited		medium		slow		stop

4. „Slow“ : Vmax 15 mph (RAILSIM allows 20 mph)						„Restricted“

...at next signal: 	clear		limited		medium		slow		stop	       (20mph or less)

5. „Stop“ : DO NOT PASS !			6. Other signal appearance		7. „clear“ (continued) :

1*)2*)3*)							1*)2*)	1*)2*)

1*) middle light can be positioned like this.	2nd next Signal shows :

2*) lower light can be missing		stop	      lim.,med. or slow

3*) middle light can be missing